SANGFOR VPN

Tunnel NAT and Tunnel Routing

Guide

SANGFOR Technologies Co., Ltd.

19.Oct 2019

Sangfor VPN Tunnel NAT and Tunnel Routing Guide

Sangfor VPN Tunnel NAT and Tunnel Routing Guide	2

1. Requirement Confirm	3

2. Solution Analysis	3

3.	Sangfor Solution: Tunnel NAT & Tunnel Routing	4

4.	Configuration Step	4

4.1 HQ side:	4

4.2 Branch side:	4

5.	Detail Configurations	5

5.1 HQ side:	5

5.2 Branch Side:	7

1. Requirement Confirm

- Branch A/B/C has the same segment, but Branch need to visit HQ server

- Branch A/B/C need to visit each other

- Customer don’t want to change branch network segment

2. Solution Analysis

- Due to standard IPSec VPN requires set both side network segment, then automatic generate a routing to VPN tunnel. If two side has the same network segment, the VPN tunnel will can’t be built.

- If branch subnet is the same with HQ, no solution.

- Sangfor Solution: Tunnel NAT & Tunnel Routing

Tunnel NAT Introduction:

NAT branch IP to a virtual IP in VPN tunnel, used for branches have network conflict issue but need to visit HQ scenario.

Tunnel Routing Introduction:

Used for branches can’t build VPN tunnel directly but branches have a same VPN service side and need to visit each other scenario, branches can visit each other via VPN service side.

- When HQ server receive requests from 10.0.0.10, we know the real requests from Branch A 192.168.0.10.

- HQ users are able to visit Branch B server 192.168.0.11 by 10.0.1.11

- Branch A user 192.168.0.12 and Branch C user 192.168.0.12 visit each other by 10.0.0.12 and 10.0.2.12

- Configuration Step

4.1 HQ side:

- Set Webagent

- Set VPN local subnet (If HQ has multiple network segment)

- Set virtual IP pool

- Create VPN user for branch and set tunnel NAT

4.2 Branch side:

- Build VPN connections

- Set VPN local subnet (If branch has multiple network segment)

- Set Tunnel routing (If branches need to visit each other)

- Detail Configurations

(The configuration may in different page in different software version, this guide based on V9.5.1)

5.1 HQ side:

- Set Webagent used for branches find VPN service IP and port

Format:

IP:4009

IP#IP:4009

webagent.sangfor.com/xxxx/xxxx (Dynamic IP scenario)

- Set VPN local subnet

Just like standard IPSec VPN Phase 2 local subnet, if site no multiple segment, no need this step.

- Set Virtual IP pool

- Create VPN user for branch and set tunnel NAT

5.2 Branch Side:

- Build VPN connections

- Set VPN local subnet

Just like standard IPSec VPN Phase 2 local subnet, if site no multiple segment, no need this step.

- Set Tunnel routing

(If Branch A just only need to visit HQ, no need to visit each other, don’t need to set Tunnel routing)